2026  Gut n go full size

GENERAL RULES AND CAR PREPARATION

- Any 1980 and newer  passenger car permitted except for the following. No specialty vehicles, checker cabs, limos, hearses, imperials, trucks, 4WD vehicles

- The vehicle you choose to run must have the same matching body style and frame. Example, You can not put a square body vic on a 1998-2002 frame.

- Drivers must be 16 years of age or older. Drivers under the age of 18 must have a parent/guardian signature and be notarized. Proof of age and ID may be requested and must have documentation on hand if requested. No passengers allowed.

- Cars must be completely stripped clean and done so prior to the show. No interior or any other flammable materials present. All wagons must remove roof racks and interior decking.All glass must be removed and swept clean.Original full tanks must be removed.

- Car must have a working seat belt and brakes at all times.

- Batteries and fuel cell must be moved to the drivers compartment (2 battery maximum)

- Drivers must have a DOT helmet, long pants, eye protection, and closed toed shoes and we highly recommend a fire suit/jacket or long sleeves

- The driver is the only one allowed in tech, no one else is allowed. We will give you a piece of paper with everything written down for you to take back with you to show your crew. You must have your paper with you for re tech.

- All rules are to the interpretation of the officials. If unsure of a rule call or text to get clarification so you understand what we want. We reserve the right to drill, scope, unbolt anything we feel could potentially be hiding something.

- Pre-run cars must follow these rules and be changed to our rule format. We will not change the rules to fit your car, you must change your car to fit our rules.

- We reserve the right to refuse any car or driver

- All added welds must be max ½” width and ¼” height. Repair plates, tilting, bumper plates, shocks brackets, etc.

- Any factory part can be interchanged as long as the car is permitted to run this class. This must be a factory bolt on part and that could be purchased from a parts store. Don’t abuse this, or try to read into it somehow. May remove metal but cannot add back or weld to make it work, must only bolt in a factory manner.

- Drivers please understand that this event is a dangerous high impact motorsport  and if you are having any kind of medical problem before the show you need to  inform any Chaos In The Valley Promotions employee.

- You will have two times through tech, if on your second attempt you do not meet

inspectors standards you will be loaded and no refund will be issued. Our job as

inspectors is to make sure it is clear and understandable of what we want done so

there shouldn’t be any questions or misunderstanding.

All Inspection decisions are final.

FRAME

- Frame must be absolutely 100% stock, no welding anywhere will be permitted unless otherwise stated.

- No frame shaping, tilting, cold bending or any other kind of framework allowed unless otherwise stated.

- Cars may be dimpled in two places per rail behind the rear axle.

- Unibody cars still have frame rails, follow the frame building rules accordingly.

- No fresh undercoating or painting of frames. Over spray is considered paint. If found you will be required to remove it.

- No seam welding even where the factory missed. If found to be welded, you will be forced to torch, grind, or cut out the weld the entire length.

- Sway bar must be 100% factory on both the frame and A arm or spindle.

- May shorten front frame to front edge of core support hole

- If it is a weld on mount leave the remaining portion of the body mount in place. If you remove the body mount completely or relocate it, you will not run.

- No seams on mount may be rewelded

- If your car does not have a weld on mount may only shorten to 1” in front of factory core support hole

- No fix it plates fresh or pre ran

BUMPERS can be changed but not mandatory

- Use bumper rules for the front and rear of the car unless specified otherwise.

- Any factory automobile bumper permitted must be stock and factory condition or just run factory one on car

- May trim ends of bumper down

- Bumper must bolt on to factory location  with factory size bolts

- Bumper height restrictions

- Front bumper height must be a maximum of 22” to bottom of the bumper

- Rear bumper height must be minimum of 15” to bottom of the bumper

- All fresh or pre run cars must meet bumper height requirement

- If bumper shock or bracket is used it must be in factory form and location. Must be from that specific car type and year and bolted only. No welding is allowed on the shock/bracket.

- You are allowed two spots of double strand 9 wire or 3/16” chain from bumper to sheet metal only. May use a standard 1” washer welded to keep sheet metal from ripping.

- At the front bumper it may only go around core support, must not be wrapped through hood or around front core support mounts

- At the rear bumper it may only go from bumper to trunk lid must only be to sheet metal

SUSPENSION

- Suspensions front and rear must be factory original to the vehicle that you are running. These must be factory stock bolt on parts and that could be purchased from a parts store. No aftermarket or modified parts allowed. No interchanging of parts.

- Front suspension must bounce no solid suspension

- Must use factory rear that comes with your car

- All control arms front or rear must be factory to the car you are running.

- Coils may be wired to rear end only

- Factory leaf spring cars can not be altered in any way. No home made clamps

- You may set your rear height by chaining it down. Chains can not be welded and can only wrap around frame in one location on each side, maximum ⅜” chain. Remember your bumper height restrictions.

- DOT TIRES ONLY ON STOCK RIMS NO MODIFICATIONS ALLOWED

BODY

- No body modifications must be stock unless otherwise stated.

- No changing of body mounts must remain factory, zero modifications.

- Sun or moon roofs must be covered using no larger than ⅛” sheet metal.

- Front fenders and rear quarter panels are not allowed to be creased . You may also cut wheel wells for tire clearance. Wheel wells may only be bolted back together using up to 5 - ⅜” standard bolts, nuts, and washers.

Doors

- All  doors may be wired or chained, one of the two options only. None of which are allowed to attach, go around or through the frame. If wired or chained only 4 spots per vertical door seam. One double strand of 9 wire or 3/16” chain

Trunks/Tailgates

- Trunks, tailgates, and speaker decks must remain in factory upright height and position above rain channels. These must also be factory trunks and tailgates for the type of car you are running.

- A simple tuck is allowed. No wedged cars allowed pre-run or fresh. Tuck may not connect to the trunk floor pan. must have two 5”x5” holes in the trunk lid. May bolt back together with 8 - ⅜” standard bolt, nut and washers.

- Trunk lid or tailgate may be wired, chained, but must only choose one method.  None of which are allowed to attach, go around, or through the frame. If wired or chained only 4 spots per trunk seam. One double strand of 9 wire or 3/16” chain

- If your car is a pre-run and your quarters are folded over you must attempt to bring them back up to factory location. Rear quarters may not be creased but tops must remain in upright factory position.

Hoods

- Hoods must remain open for tech or we will not inspect your car.

- Hoods must have a minimum 12”x12” hole in it for fires. It can be bolted back together with 8 - ⅜” standard bolt, nut and washers.

- Hoods may be chained or wired in 6 spots. Must only choose one method.

- If using chain or wire you are allowed one double strand of 9 wire or 3/16” chain

Window Bars

- The front windshield must have One double strand of 9 wire or 3/16”

- The rear windshield is allowed 1 double strand of 9 wire or 3/16” chain from roof to speaker deck. Must only be bolted and all links must be free floating. Standard 1” washer welded to keep sheet metal from ripping. No rear window bar on wagons.

- If you cut out the center of your firewall you are not permitted to cap the ends..

CAGE recommend but not mandatory run at your own risk

- A 4 point cage with a halo bar is highly recommended but is optional. Cage size must not exceed 6”x6” total diameter or the equivalent of this dimension.

- Cage material can not run through the doors

- All cage material must be 5” off all floor sheet metal, no exceptions.

- Dash bar must be 5” off the firewall and 5” off the trans tunnel. Rear seat bar may not be any farther back then where the rear kick panel meets the floor (where the front of the back seat would be located)

- Side bars may be a maximum of 62” long and must also remain 5” away from the fire wall and 5” from all floor sheet metal.

- Halo bars are allowed, must be straight up and down and connect to the rear seat or side bars. It may be connected to the roof in two spots. Attachment points may be 2”x2”x6” long angle with a ½” bolt with a standard washer and nut only

- Gussets in the corners of the cage are allowed, 4”x¼” material

- You may also have a center bar

- No fuel cell protector if running a holder must be 4 inches away from all materials

DRIVETRAIN

- Must run factory engine and transmission for that model of car

- Engine mounts must be stock

- you may have two chains one on each side of the engine. May wrap frame one spot but must be free floating

- No front plates,cradles , pulley protectors, distributor protectors, carb halos, transmission braces, coil pack protectors, or mid plates allowed.

- Transmissions must be factory OEM cases, no ultra bells or metal added.

- Transmission crossmembers must be factory to your car and mounted in factory location with OEM mounting only. No chaining transmissions.

- No oil or transmission coolers allowed, hoses/lines must be looped.

NO AFTERMARKET PARTS MUST BE FACTORY ORIGINAL ETC YOU GET THE POINT

All rule and event questions please direct to matthew kingree (540) 335-6439 all questions must be asked via text including your name,show your inquiring about, and car number if phone call is needed it will be recorded or followed with a detailed text.

Text and calls will be only made monday-thursday 5:00pm - 9:00pm and friday-sunday 10:00am-9:00pm est time