CHAOS IN THE VALLEY PROMOTIONS LLC

2026 STOCK FULL SIZE TRUCK

General rules/ car preparations

- Any factory 2 wheel drive truck,suv, and van

- If 4 wheel drive one of the drive shafts or axles must be removed

- Cars  Must be complete stripped clean no trim interior or any other flammable materials must be removed prior to the show all wagons must remove roof racks and decking

- All glass must be removed and swept clean

- Original full tanks must be removed and placed in back seat area

- All batteries must be moved in the drivers compartment ( 2 battery maximum)

- Drivers must have a dot helmet,long pants,eye protection, and closed toed shoes we highly recommend a fire jacket/suite or long sleeves

- Car must have a seat belt and working brakes at all times

- You will have two times threw tech if on your second attempt you did not meet satisfaction to what the inspectors wanted you will be loaded and no refund will be issued ( as our job of inspections we will make sure it is clear and understandable of what we want done so there shouldn’t be any questions or miss understanding)

- Drivers only in tech no one else is allowed   ( we will give you a piece of paper with everything written down for you to take back with you to show your crew) must have paper with you for re tech

- All rules are to the interpretation to the officials ( if unsure of a rules call or text to get clarification so you understand what we want

- All judge’s decision are final

- We reserve the right to drill,scope,unbolt anything we feel could potentially be hiding something

- Pre ran cars must follow these rules and change what needs to be changed we will not change the rules to fit your car you must change your car to fit our rules

- We reserve the right to refuse any car/ driver

- Drivers under the age of 18 must be 16 years and older with parent/guardian signature and be notarized proof of id may be asked for

- All added welds must be max ½ inch wide and ¼ inch tall (repair plates,tilting,bumper plates,shocks brackets etc)

- Drivers please understand that this event can be dangerous and if your having any kind of medical problems before the show to inform any Chaos In The Valley Promotions employee

Frame

- Frames must be stock no welding unless otherwise stated

- seams may be welded

- Frames can be shortened to core support

- All frame holes must remain open

Repair/prior plates/9 wire rules

- If car is fresh/ pre ran you are allowed 4- 4x4” ¼” square plates (2 per rail)

- Square plates only

- No parallelogram plates

- Must have a 1 inch gap between welds

- May not fill 1 inch gap with any extra weld

- 1/2 inch weld max around plate

- Welds/plate Must remain 2 inches away from bumper shock/ bracket or 6”x4” plate

- Welds/plate must remain 1in away from a arm strap

- May have 4 spots of double strand 9 wire from sheet metal to frame May not connect or go around any part of cage

- Plates must be external

- Plates Must be painted bright color for easy identification

- Welds must be painted orange where cars are restubed

- Plates are not allowed to tie any engine cradles/ midplates/ block savers/ transmission crossmember into frames.

- Behind rear of car You are permitted 1 double strand of #9 wire or May use a standard ⅜ chain must wrap around frame one time or go through a factory whole no metal may be added to frame

- You will not be given any extra plates so use wisely

Body

- All body bolts must be factory including core support bolts

- Drivers door may be welded solid all other doors can be welded with the 5 on 5 method (put a plate skip 5 inches put other plate) or may use chain or 9 wire

- Tops of doors may be welded

- Bed sides may be welded to cab in two 3x3 locations

- Bed sides must remain factory height

- Tailgates may be welded with total of 4 plates or may use 8 spots of chain and 9 wire sheet metal to sheet metal must pick one way not both

- Hoods may be bolted down with a total of 6 spots 2x2 angle 4 inch long with ⅜ bolt max or you may have chain or 9 wire in 8 spots

- All hoods must have a 12x12 hole for fires may be bolted back with 4 ⅜ bolts

Cage

- A 4 point cage and halo bar  is highly recommend but optional

- your dash bar must be 5 in off the firewall and 5 in off the trans tunnel your rear seat bar may not be any farther back then where the rear kick panel meets the floor (where the front of the back seat would be located)

- May have a center bar must be 5in off all parts of the floor sheet metal

- DOWN BARS will be allowed must be mounted in cab and may be to frame only

- Hallo bars are allowed must be straight up and down and connect  to rear seat or side bars it also may be connected to the roof in two locations only with 3 in long 2x2 angel with a ½ in bolt maximum

- You will be allowed a gas tank protector 32in wide maximum it will be allowed to touch the package tray only can be bolted or welded if you build a halo over the back half of your tank it can not be attached to roof

Suspension

- Must be all factory components no aftermarket spindles or ball joints must be all OEM parts if using a weld in style ball joint it must have limited fabrication done No aftermarket springs

- No ball joint protector

- May use aftermarket steering columns

- No Aftermarket or reinforced tie rods

- You may lock front suspension you may weld strut or use a 2x4 ¼ plate front and back side of a arm

- Shocks must be factory

- Trucks Must use factory appearing rears and can not be attached to frame other then where factory intended

- You may chain the rear down but rear bumper height must remain 16 inches tall

- no coil to leaf conversion

- Factory leaf spring vehicles with factory leafs  may have 4 total clamps on each leaf ( 2in wide 4 in long 5/16 bolt do not put tape around leafs)

- You may wire or weld your coil springs to rear do not weld springs to frame in anyway

- Any tires will be permitted black and round

- You may have full center rims and a valve stem protector, lip guard protector can only be 1in wide will be allowed bead locks will be permitted

Bumpers

- Any factory loaded seam welded automobile bumper

- Use bumper rules for front and rear of car

- we will be allowing a smith metal works or similar bumpers

- Home made bumpers not to exceed 8x8 tube may have a point must not be no more than 4in away from front face of bumper and must expand 32 inches across the front

- Rear bumpers must be a minimum of 5 inches on back surface

- Replicated  bumpers will be allowed

- No pointed or slanted bumpers on rear of any car ( chrysler pointys replica or factory)

Bumper mounting

- Bumpers may be welded on must adhere to following rules

- Bumper may be hard nosed

- If hard nosed factory shock/bracket must be removed

- May use a 4”x6”x¼ inch flat plate to help attach bumper must touch bumper and only be on one side of the frame not all 4

- If bumper shock/bracket is used must be in factory form and location and must be bolted no welding allowed on shock/ bracket  and will not be allowed the 4”x6” plate

- Brackets/shock must be from that car specific  no changing models years or makes

- no alternating brackets or shocks

- ½ inch weld max around plate

- Measurement will be from back of bumper

- Bumpers may be welded on to the end of the frame no sliding bumper over or inside of end of the frame or subframe rails

- May use a 6”x6” ¼”  plate welded to end of frame to make a flat mounting surface must be welded in between the fame and bumper and remain the 6”x6” no folding over plate on to frame

- If unable to weld your bumper on you may wire, and chain

- You are allowed two spots of double strand  9 wire from bumper to sheet metal only must follow the following

- On the front bumper  9 wire may only go around core support may not be wrapped through hood or around front all thread mounts

- Rear bumper 9 wire may only go from bumper to trunk lid must only be to sheet metal

Bumper height restrictions

- Front bumper height is 22 inches to bottom of the bumper

- Rear bumper height is 20 inches to the bottom of the frame around the bumper mounting area.

- All trucks/suvs will meet bumper height requirements by our inspection area and all cars fresh or pre ran will meet requirement

Motor and trans

- Any motor or transmission combo

- Motor mounts may be changed to any factory or rubber style mount

- no distributor protectors no coil protectors

- No Lower engine cradles permitted

- No trans braces will be allowed

- Trans mounts must be factory you may chain trans around the mount

- Trans and oil coolers are permitted but must be mounted in drivers compartment and must use appropriate lines ( hydraulic hoses are highly recommended)

- Any metal fuel cells permitted or boat tanks if its a plastic tank it must be in some type of  metal structure   (we all have families that need us this rule is the most important and will be enforced no matter the outcome )

- If tank is in front of rear axle you may leave it there and must have a skid plate

- All fuel lines need to be doubled clamped

- May have two engine chains one on each side of the motor may only weld to engine crossmember not to side rails only one link may be weld all others must be free floating ⅜ chain max

All rule and event questions please direct to matthew kingree (540) 335-6439 all questions must be asked via text including your name,show your inquiring about, and car number if phone call is needed it will be recorded or followed with a detailed text.

Text and calls will be only made monday-thursday 5:00pm - 9:00pm and friday-sunday 10:00am-9:00pm est time