2026 Chaos in the valley  youth compacts

General rules/ car preparations

Any year car permitted front wheel drive cars only within 110 inch wheelbase

- Drivers must be 12-16 without a driver's license/permit

- If you are between the ages of 15-16 you do not have to have a passenger

- Proof of age may be requested and must have documentation on hand if requested

- If you have drove in a adult class you forfeit your youth age and must move up a class

- Must be complete stripped clean no trim interior or any other flammable materials must

- be removed prior to the show all wagons must remove roof racks and decking

- All glass must be removed and swept clean

- Original full tanks may be removed and placed in back seat area

- All batteries must be moved in the drivers compartment ( 2 battery maximum)

- Drivers must have a dot helmet,long pants,eye protection, and closed toed shoes we highly recommend a fire jacket/suite or long sleeves

- Car must have a seat belt and working brakes at all times

- Absolutely no spray foam in any shape or form allowed

- All sun roofs must be covered

- Drivers only in tech no one else is allowed.

- If car is pre ran from a previous show you will undergo a fresh and new inspection before every show anything that is found under a new inspection must and will be removed to participate in present event

- All rules are to the interpretation to the officials ( if unsure of a rules call or text to get clarification so you understand what we want)

- All judge’s decision are final

- We reserve the right to drill,scope,unbolt anything we feel could potentially be hiding something

- Pre ran cars must follow these rules and change what needs to be changed we will not change the rules to fit your car you must change your car to fit our rules

- We reserve the right to refuse any car/ driver

- All added welds must be max ½ inch wide and ¼ inch tall (repair plates,tilting,bumper plates,shocks brackets etc)

- Drivers please understand that this event can be dangerous and if your having any kind of medical problems before the show to inform any Chaos In The Valley Promotions employee

- You will have two times through tech, if on your second attempt you do not meet

inspectors standards you will be loaded and no refund will be issued. Our job as

inspectors is to make sure it is clear and understandable of what we want done so

there shouldn’t be any questions or misunderstanding.

All Inspection decisions are final.

Frames

- Frames must be 100 percent stock absolutely no welding anywhere will be permitted unless otherwise stated

- No seam welding even where factory missed

- If found you will cut the center of weld out with a grinder or torch

- You may notch or dimple cars in one location

- UNIBODY CARS STILL HAVE FRAME RAILS, FOLLOW THE FRAME BUILDING RULES.

- No painting or undercoating of frames

- If found you will be required to remove

- Over spray is considered paint and will be removed also

- No fresh undercoating

- Failure to remove could result in an automatic load

- If frame is rusted we must have proof of rust and must be approved by matthew kingree approved times and ways of contact  located at end of rules

Shorting of frames

- May shorten frame to front edge of Core support

- No shorting of rear of vehicles

Repair/fresh plates/9 wire

- If car is fresh/pre ran you are allowed 4- 4x4” ¼” square plates (2 per rail)

- Square or parallelogram plates

- Must have a 1 inch gap between welds

- Welds/plate Must remain 1 inches away from bumper shock/ bracket

- May have 8 spots of double strand 9 wire

- Plates must be external

- Must be painted orange for easy identification

- Plates are not allowed to tie subframe to uni body or k frames to uni body

- No 9 wire or chain may be ran from subframe to  k frame

Bumpers

- Any factory loaded seam welded automobile bumper

- No tube bumper at all automatic load

- Use bumper rules for front and rear of car

- No tube bumpers on youth cars

- Rear bumpers must be a minimum of 5 inches on back surface

- Replicated  bumpers will be allowed except pointy replicas or factory chrysler pointy

- Replica bumpers must follow same specs as a bumper legal in this class

- No pointy or slanted bumpers on any car

Bumper mounting

- You will be allowed a 4 inch wide by 4 inch long bumper bracket (4”x4”x ⅜”)

- Plate can only be on one side of the frame not all 4

- May contour or remain straight

- Must touch front bumper and anything past 4 inches will be removed factory or not

- If you choose to run a factory shock or bracket anything pass the 4 inches will be removed

- If using the plate options you must remove factory shock/ bracket

- Any thing on inside of frame will be automatic load

- Bumper brackets and shocks must be factory to car you are running and bumper shocks must be in the factory location

- May use a 6’’x6’’ ¼  plate welded to end of frame to make a flat mounting surface must be welded in between the fame and bumper and remain the 6’’x6’’ no folding over plate on to frame

- Bumpers may be welded on to the end of the frame no sliding bumper over or inside of end of the frame or subframe rails

- If unable to weld your bumper on you may wire, chain,and bolt your bumper on and may leave the shock this is for the cars that bumper was not welded on

- You are allowed two spots of double strand  9 wire from bumper to sheet metal only must follow the following

- On the front bumper  9 wire may only go around core support may not be wrapped through hood or around front all thread mounts

- Rear bumper 9 wire may only go from bumper to trunk lid must only be to sheet metal

Body

- No body modifications must be stock unless otherwise stated

- No changing of body mounts must remain factory zero modifications

- Creasing is allowed but limited to front fenders and rear quarters

- Sun roofs must be covered

Doors

- Drivers door and front passengers door may be welded solid  and all other doors may be welded with a total of 6 plates 3x5x1/4

- An inner or outer door plate on drivers door and passengers door is highly recommended.

- No thicker than ¼ inch and can only go 3 inches of each door seam

- If not welded  may be chained or wired  8 spots per door

- you may weld a standard ⅝ washer around holes to prevent ripping

trunks/tailgates

- Rear quarters must remain factory height

- Trunks must remain factory and above rain channels you may do a simple tuck but can not attach to the floor in any way

- Trunks May be weld/wired/chained/ bolted not all 4

- If welded 6 spots 3x5x1/8 plates

- If chained or wired you may do  8 spots you

- may weld a standard ⅝ washer around holes to prevent ripping

- you may use 2’’x2’’ angle 6 inches long in 6 spots with 2 standard ½ in bolt and washer

Hoods

- Hoods must have a minimum 12x12 hole in it for fires can be bolted back together with 4 ⅜ bolts with a standard nut and washer

- hoods must remain open for tech

- hoods may be chained, wired or bolted  one of the three may be used

- If chained or wired you may do 8 spots of wire or chain you may weld a standard ⅝ washer around holes to prevent ripping

- If bolted you may use 6 spots and you may use 2x2 angle 2 inches long in 6 spots with a standard ½  bolt and washer

Window bars

- You may have one front and rear window bar

- Front window bar may run from halo bar to first 6 inches or cowl panel or right to top of dash bar

- If its square tube nothing bigger than 2x2 ¼ pipe

- 3in wide ⅜ stock bar

- Chain

- Or 9 wire

- One of the following not all of them

- Rear window bar can only attach to first 6 inches of roof and first 6 inches trunk lid closest to window

- If its square tube nothing bigger than 2x2 ¼ pipe

- 3in wide ⅜ stock bar

- Chain

- Or 9 wire

- One of the following not all of them

- Rear window bar must be 6 inches away from gas tank protector

- No bars may attach rear window bar to gas tank protector

- No rear window bars on wagons

Cage

- A 4 point cage and halo bar  is highly recommend but is optional

- Cage size may not exceed 2 x 6 ¼ or 4 inches in diameter

- Cage can not be in the doors

- All cage material must be 5 inches off floor no exceptions

- your dash bar must be 5 in off the firewall and 5 in off the trans tunnel your rear seat bar may not be any farther back then where the rear kick panel meets the floor (where the front of the back seat would be located)

- Side bars must be 62 inches long and must also remain 5 inches away from fire wall and 5 inches from floor

- May have a center bar must be 5in off all parts of the floor sheet metal

- Hallo bars are allowed must be straight up and down and connect  to rear seat or side bars it may  be connected to the roof in two spots

- Attachment points may be 2x2 ¼ 6 inch long angle with a ½ bolt with a standard washer and nut  only

Gas tank protector

- You will be allowed a 32in wide tank protector

- Can be no farther back than the speaker deck on sedans and no farther back than the rear interior door seam on wagons

- It can not attach to any sheet metal

- You are allowed to build an upper halo around your gas tank protector but can be no higher than rear speaker deck on sedans and remain 15 inches away from roofs on wagons

- You may have bars running from tank protector to vertical up rights on halo bar

- If your tank is under your car you may not have a tank protector

- No bars may attach gas tank protector to rear window bar

suspension

- Must remain stock absolutely no aftermarket or modified parts 100% stock

- Only aftermarket parts your allowed is spring spacers or the strut risers

- Must be bolt on part and remain on top side of strut this goes for front and rear

- Cars must have suspension bounce

- All control arms must be stock and no modifications allowed and must be in factory location of the car you are running

- No chaining of the rear to set the height

- Any aftermarket trailing arms will be removed and replaced with factory ones failure to do so may result in an automatic load

Tires

- Any tire and wheel combo permitted

- Must have started as a factory rim

- May run bead locks, lip proctors, full centers,valve stem protectors

motor/trans

- Motor and trans must be factory to make and model no switching makes ( ford to ford chevy to chevy Toyota to Toyota)

- You may run an aftermarket motor mounts on front of engine only 2x2 max size

- Toyota products may change the front motor mount running to the k frame must bolt back as factory intended no welding allowed

- Gm products may change the upper two mounts must bolt in to the factory brackets on both motor and core support

- Honda products may change the front mount also must be done as factory attend

- No engine cradles distributor protectors trans braces coil pack protectors no protectors in general must all be oem

- Trans must be factory to car and mounted in factory location

- Trans coolers are permitted but must be mounted in drivers compartment and must use appropriate lines ( hydraulic hoses are highly recommended)

- You may run aftermarket shifter

- If tank is under the car and in front of the rear axle you may leave it there but must have metal skid plate

- Headers will be allowed

- Carb conversions are allowed

Full tanks

- Any metal fuel cells permitted or boat tanks if its a plastic tank it must be in some type of  metal structure

- Must be secured and safe if we deem any part of fuel system is unsafe you will fix issues until it’s deemed safe

- (we all have families that need us this rule is the most important and will be enforced no matter the outcome )

- If tank is in front of rear axle it may be left there but must have a skid plate

Aftermarket parts permitted

- Headers

- Shifters

- Pedal combos

- Steering columns

- Fuel cells

For Tech and approval questions you must text Matthew Kingree 540-335-6439. Include your name and show your inquiry about. Mon-Thur 5-9pm Fri-Sat 10am-9pm